Eco Audit Report.. Product name.; Cross Laminated Timber.. Country of manufacture.; World... Country of use.; World.. Product life (years).; 20.. Summary:.... Energy details.; CO2 footprint details.; Cost details.. Phase; Energy (MJ); Energy (%); CO2 footprint (kg); CO2 footprint (%); Cost (GBP); Cost (%). Material; 2.82e+03; 39.2; 94.5; 10.2; 226; 59.3. Manufacture; 4.36e+03; 60.6; 829; 89.7; 153; 40.1. Transport; 8.36; 0.1; 0.602; 0.1; 2.22; 0.581. Use; 0; 0.0; 0; 0.0; 0; 0. Disposal; 3.1; 0.0; 0.217; 0.0; 0.053; 0.0139. Total (for first life); 7.2e+03; 100; 924; 100; 382; 100. End of life potential; -474; -1.61..

|  | Eco Audit Report.. Summary.. Energy Analysis.... Energy (MJ/year). Equivalent annual environmental burden (averaged over 20 year product life):; 360... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Energy (MJ); %. Timber; Pau marfim (t); Virgin (0%); 5; 1; 5.6; 6.1e+02; 21.5. laminate; Epoxy/Aramid, PCB laminate; Virgin (0%); 6; 1; 6; 2.2e+03; 78.5. Total; 2; 12; 2.8e+03; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; Energy (MJ); %. Timber; Cutting and trimming; 10; 0.56; kg; 0.17; 0.0. laminate; Resin transfer molding (RTM); -; 6; kg; 77; 1.8. Pressing; Adhesives, cold curing; -; 4.3e+02; m^2; 4.3e+03; 98.2. Total; 4.4e+03; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); Energy (MJ); %. road; 55 tonne (8 axle) truck; 1e+03; 7.8; 93.4. ship; Ocean freight; 2.8e+02; 0.55; 6.6. Total; 1.3e+03; 8.4; 100.. Breakdown by components.. Component; Mass (kg); Energy (MJ); %. Timber; 5; 3.8; 45.5. laminate; 6; 4.6; 54.5. Total; 11; 8.4; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Energy (MJ); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Energy (MJ); %. Timber; Re-manufacture; 80.0; 1; 32.3. laminate; Downcycle; 50.0; 2.1; 67.7. Total; 3.1; 100.. EoL potential:.. Component; End of life option; % recovered; Energy (MJ); %. Timber; Re-manufacture; 80.0; -4.7e+02; 100.0. laminate; Downcycle; 50.0; 0; 0.0. Total; -4.7e+02; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. CO2 Footprint Analysis.... CO2 (kg/year). Equivalent annual environmental burden (averaged over 20 year product life):; 46.2... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); CO2 footprint (kg); %. Timber; Pau marfim (t); Virgin (0%); 5; 1; 5.6; 3.1; 3.2. laminate; Epoxy/Aramid, PCB laminate; Virgin (0%); 6; 1; 6; 91; 96.8. Total; 2; 12; 94; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; CO2 footprint (kg); %. Timber; Cutting and trimming; 10; 0.56; kg; 0.013; 0.0. laminate; Resin transfer molding (RTM); -; 6; kg; 6.2; 0.7. Pressing; Adhesives, cold curing; -; 4.3e+02; m^2; 8.2e+02; 99.3. Total; 8.3e+02; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); CO2 footprint (kg); %. road; 55 tonne (8 axle) truck; 1e+03; 0.56; 93.4. ship; Ocean freight; 2.8e+02; 0.04; 6.6. Total; 1.3e+03; 0.6; 100.. Breakdown by components.. Component; Mass (kg); CO2 footprint (kg); %. Timber; 5; 0.27; 45.5. laminate; 6; 0.33; 54.5. Total; 11; 0.6; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; CO2 footprint (kg); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; CO2 footprint (kg); %. Timber; Re-manufacture; 80.0; 0.07; 32.3. laminate; Downcycle; 50.0; 0.15; 67.7. Total; 0.22; 100.. EoL potential:.. Component; End of life option; % recovered; CO2 footprint (kg); %. Timber; Re-manufacture; 80.0; -1.6; 100.0. laminate; Downcycle; 50.0; 0; 0.0. Total; -1.6; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. Cost Analysis.... Cost (GBP/year). Equivalent annual environmental burden (averaged over 20 year product life):; 19.1... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Cost (GBP); %. Timber; Pau marfim (t); Virgin (0%); 5; 1; 5.6; 35; 15.6. laminate; Epoxy/Aramid, PCB laminate; Virgin (0%); 6; 1; 6; 1.9e+02; 84.4. Total; 2; 12; 2.3e+02; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Country of manufacture.; World.. Component; Process; Length (m); % Removed; Amount processed; Cost (GBP); %. Timber; Cutting and trimming; -; 10; 0.56; kg; 0.0011; 0.0. laminate; Resin transfer molding (RTM); -; -; 6; kg; 21; 14.0. Pressing; Adhesives, cold curing; -; -; 4.3e+02; m^2; 1.3e+02; 86.0. Total; 1.5e+02; 100... Transport:.; Summary.. Package dimensions.. Height (m); Width (m); Depth (m). 0.1; 0.2; 0.4... Breakdown by transport stage.. Stage name; Transport type; Distance (km); Cost (GBP); %. road; 55 tonne (8 axle) truck; 1e+03; 1.1; 49.9. ship; Ocean freight; 2.8e+02; 1.1; 50.1. Total; 1.3e+03; 2.2; 100.. Breakdown by components.. Component; Mass (kg); Cost (GBP); %. Timber; 5; 1; 45.5. laminate; 6; 1.2; 54.5. Total; 11; 2.2; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Cost (GBP); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Cost (GBP); %. Timber; Re-manufacture; 80.0; 0.024; 45.5. laminate; Downcycle; 50.0; 0.029; 54.5. Total; 0.053; 100... Notes:.; Summary.... |
| --- | --- |

Eco Audit Report.. Product name.; Hempcrete.. Country of manufacture.; World... Country of use.; United Kingdom.. Product life (years).; 100.. Summary:.... Energy details.; CO2 footprint details.; Cost details.. Phase; Energy (MJ); Energy (%); CO2 footprint (kg); CO2 footprint (%); Cost (GBP); Cost (%). Material; 57.5; 2.0; 9.2; 4.1; 4.96; 20.6. Manufacture; 2.85e+03; 97.9; 214; 95.8; 18.2; 75.7. Transport; 1.86; 0.1; 0.134; 0.1; 0.871; 3.62. Use; 0; 0.0; 0; 0.0; 0; 0. Disposal; 0.92; 0.0; 0.0644; 0.0; 0.0222; 0.0921. Total (for first life); 2.91e+03; 100; 223; 100; 24.1; 100. End of life potential; -43.2; -6.9..

|  | Eco Audit Report.. Summary.. Energy Analysis.... Energy (MJ/year). Equivalent annual environmental burden (averaged over 100 year product life):; 29.1... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Energy (MJ); %. Hempcrete block; Hemp fiber; Virgin (0%); 4.6; 1; 5.8; 58; 100.0. Total; 1; 5.8; 58; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; Energy (MJ); %. Hempcrete block; Cutting and trimming; 20; 1.2; kg; 0.35; 0.0. Hydraulic pressing; Construction; -; 2.9e+04; kg; 2.9e+03; 100.0. Total; 2.9e+03; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); Energy (MJ); %. ship; Ocean freight; 2.8e+02; 0.23; 12.4. land; 55 tonne (8 axle) truck; 5e+02; 1.6; 87.6. Total; 7.8e+02; 1.9; 100.. Breakdown by components.. Component; Mass (kg); Energy (MJ); %. Hempcrete block; 4.6; 1.9; 100.0. Total; 4.6; 1.9; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Energy (MJ); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Energy (MJ); %. Hempcrete block; Reuse; 75.0; 0.92; 100.0. Total; 0.92; 100.. EoL potential:.. Component; End of life option; % recovered; Energy (MJ); %. Hempcrete block; Reuse; 75.0; -43; 100.0. Total; -43; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. CO2 Footprint Analysis.... CO2 (kg/year). Equivalent annual environmental burden (averaged over 100 year product life):; 2.23... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); CO2 footprint (kg); %. Hempcrete block; Hemp fiber; Virgin (0%); 4.6; 1; 5.8; 9.2; 100.0. Total; 1; 5.8; 9.2; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Component; Process; % Removed; Amount processed; CO2 footprint (kg); %. Hempcrete block; Cutting and trimming; 20; 1.2; kg; 0.026; 0.0. Hydraulic pressing; Construction; -; 2.9e+04; kg; 2.1e+02; 100.0. Total; 2.1e+02; 100... Transport:.; Summary.. Breakdown by transport stage.. Stage name; Transport type; Distance (km); CO2 footprint (kg); %. ship; Ocean freight; 2.8e+02; 0.017; 12.4. land; 55 tonne (8 axle) truck; 5e+02; 0.12; 87.6. Total; 7.8e+02; 0.13; 100.. Breakdown by components.. Component; Mass (kg); CO2 footprint (kg); %. Hempcrete block; 4.6; 0.13; 100.0. Total; 4.6; 0.13; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; CO2 footprint (kg); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; CO2 footprint (kg); %. Hempcrete block; Reuse; 75.0; 0.064; 100.0. Total; 0.064; 100.. EoL potential:.. Component; End of life option; % recovered; CO2 footprint (kg); %. Hempcrete block; Reuse; 75.0; -6.9; 100.0. Total; -6.9; 100... Notes:.; Summary... |
| --- | --- |

|  | Eco Audit Report.. Summary.. Cost Analysis.... Cost (GBP/year). Equivalent annual environmental burden (averaged over 100 year product life):; 0.241... Detailed breakdown of individual life phases.. Material:.; Summary.. Component; Material; Recycled content* (%); Part mass (kg); Qty.; Total mass processed** (kg); Cost (GBP); %. Hempcrete block; Hemp fiber; Virgin (0%); 4.6; 1; 5.8; 5; 100.0. Total; 1; 5.8; 5; 100.. *Typical: Includes 'recycle fraction in current supply'.. **Where applicable, includes material mass removed by secondary processes.. ***User-defined material... Manufacture:.; Summary.. Country of manufacture.; World.. Component; Process; Length (m); % Removed; Amount processed; Cost (GBP); %. Hempcrete block; Cutting and trimming; -; 20; 1.2; kg; 0.0022; 0.0. Hydraulic pressing; Construction; -; -; 2.9e+04; kg; 18; 100.0. Total; 18; 100... Transport:.; Summary.. Package dimensions.. Height (m); Width (m); Depth (m). 0.6; 0.2; 0.2... Breakdown by transport stage.. Stage name; Transport type; Distance (km); Cost (GBP); %. ship; Ocean freight; 2.8e+02; 0.46; 53.3. land; 55 tonne (8 axle) truck; 5e+02; 0.41; 46.7. Total; 7.8e+02; 0.87; 100.. Breakdown by components.. Component; Mass (kg); Cost (GBP); %. Hempcrete block; 4.6; 0.87; 100.0. Total; 4.6; 0.87; 100... Use:.; Summary.. Relative contribution of static and mobile modes.. Mode; Cost (GBP); %. Static; 0. Mobile; 0. Total; 0; 100... Disposal:.; Summary.. Component; End of life option; % recovered; Cost (GBP); %. Hempcrete block; Reuse; 75.0; 0.022; 100.0. Total; 0.022; 100... Notes:.; Summary.... |
| --- | --- |